AE 721 Report 4 Required Contents

- Due Monday 7 October 2024  9am

by e-mail to <kuaerodesign@gmail.com>
AE721_Report3_Studentlastname.docx	   example: AE721_Report3_Studentlastname.docx

AE721_Report3_Studentlastname.pdf 	   example: AE721_Report3_Studentlastname.pdf

AE721_Report3_studentlastname.m, and any other files for computation

Individual Assignment

Note that the work to be performed for this report is to be done individually. Students may not copy parts of others’ works.

- Total Points: 50

What follows is a specification for the minimum required contents of the AE 521 Report Block number 1. Your report must be organized in this manner. You may, within each section, add sub-sections if you so desire.  Each chapter must start with a statement of purpose, except the Introduction and the Summary/Conclusions chapters.

- Report Title: AE 721 Report 4 Ballistics Code Design of BASS Round

Cover - This is a good idea for both the .doc and .pdf files as it will be necessary when the file is printed to a hard copy.  Name all team members.

Title Page - This page should be the first page of the report (if a cover is used, then it will be within the cover page). This page should be numbered as page "i."

Acknowledgments – Optional, make them real. Thank Mom/Dad/HS Teachers/Pets as you feel appropriate.

Table of Contents - The table of contents shall be structured as follows:

Table of Contents                                 page

List of Symbols	iii

Acknowledgment	iv

1. Introduction	1

2. Chapter 2 Major Heading	3

2.1 Chapter 2 First Section	3

2.2 Chapter 2 Second Section	5

2.2.1 Chapter 2 First Subsection of Second Section	5

etc.

Page Numbering - The first section of the report containing the list of symbols, acknowledgment, list of tables and list of figures etc. shall be numbered with lower case Roman numerals. The body shall be numbered sequentially with Arabic numerals 1, 2, 3, etc. The Appendices shall be numbered with the letter of the appendix first followed by a dash and a page number: A-1, A-2, A-3; B-1, B-2, B-3 etc.

List of Symbols - You must include any symbols that are used in this report. You must give the names of the symbols and the units. After the main body of the symbols, you should include Greek symbols, then subscripts, followed by acronyms.

List of Figures - List any and all figures that are found in the report along with the page they are found on.

List of Tables - List any and all tables that are found in the report along with the page they are found on.

Notes on Generated Codes and Plots

Any suitable code may be used (like Excel, MatLab, other coding language). The original code must be turned in with the homework.

1. Introduction:

Describe the basic motivation for BASS ammunition. Depending on caliber, come up with your own line of reasoning. Below 15mm is small arms, 15 – 60mm is mid-caliber and typically used for different things than small arms.

2. Ballistic Modeling

Describe your code and any modifications that you made to your code. Reference the precise code that you will use line-by-line in Appendix A.

3. Assigned Round Calibers and Baseline Cartridge Design

Use your assigned calibers below to determine the form factor for the entire cartridge (total length, slug, tail etc.) and lay out the reference cartridge. Make a CAD of the reference cartridge scaling a PGU-28.

|  | Caliber (mm) | Muzzle Velocity, Vm (m/s) |  | Caliber (mm) | Muzzle Velocity, Vm (m/s) |
| --- | --- | --- | --- | --- | --- |
| Ackerley,Tyler | 6 | 823 | Magpantay,Aizaac | 22 | 1041 |
| Albalushi,Younis | 5 | 754 | Marshall,Ainsley A | 13 | 1165 |
| Allen,Charles T | 11 | 473 | Maurer,Matthew | 23 | 1192 |
| Baruth,Adam C | 5 | 533 | Morris,Connor E | 18 | 722 |
| Bernart,Gabriel | 11 | 1068 | Norsby,Zachary J | 21 | 1037 |
| Brotherton,Paetton | 12 | 699 | Olson,Hannah A | 8 | 763 |
| Caudillo,Olivia | 6 | 679 | Pankiewicz,William | 8 | 1156 |
| Cohen,Benjamin | 14 | 648 | Powers,Matthew | 6 | 981 |
| Duke,Kenny | 6 | 517 | Prendergast,Mark A | 15 | 843 |
| Garmon,Roger | 4 | 542 | Prescott,Caleb J | 20 | 559 |
| Gordon,Garrett C | 5 | 1065 | Reida,Reanne N | 9 | 804 |
| Hantla,Avery | 4 | 517 | Relan,Jennifer | 15 | 1098 |
| Hovey,Ryan W | 9 | 431 | Richardson,Carson | 19 | 916 |
| Husband,Patrick | 17 | 1022 | Rodriguez,Alex | 4 | 784 |
| Jamal,Subhan | 10 | 896 | Sollars,Reece S | 5 | 410 |
| Lawhorn,Colby D | 8 | 526 | Soto Baron,Kevin | 11 | 804 |
| Lee,Devin J | 17 | 561 | Willhite,William | 20 | 697 |
| Li,Yuwen | 12 | 870 |  |  |  |

4. Design Three Optimized Families of BASS Rounds

Assume the same muzzle energy as your baseline round of HW 3 and Chapter 3 (above).

Assume that 92% of that muzzle energy will be invested in the tungsten BASS round. The remaining 8% will, necessarily be invested in the BASS sabot.

Using your code, assuming a PGU-28 drag profile, determine what the maximum range of the reference round above given at 15° gun elevation angle firing on level ground with no winds.

Given this fixed muzzle energy, you will optimize your round design and gun elevation angle to arrive at a better design, working to optimize: i.) Time of Flight, ii.) KE on Impact, iii.) Cost given same KE on impact of reference round.

4.1 Minimal of Time of Flight BASS Design

Sweep through a range of BASS round designs, reducing flechette diameter, using tungsten as the major mass component of the BASS round. Constrain your BASS design by assuming it occupies no more than 85% of the entire reference cartridge length, has a tail stock that is at least as long as the nose and goes no faster than Mach 10. Iterate about gun elevation angle, muzzle velocity, BASS round geometry, working to find that design which goes fastest to the prescribed range at STP.

4.2 Maximum KE BASS Design

Sweep through a range of BASS round designs, reducing flechette diameter, using tungsten as the major mass component of the BASS round. Constrain your BASS design by assuming it occupies no more than 85% of the entire reference cartridge length, has a tail stock that is at least as long as the nose and goes no faster than Mach 10. Iterate about gun elevation angle, muzzle velocity, BASS round geometry, working to find that design which has the highest KE at the prescribed range at STP.

4.3 Lowest Cost BASS Design

Sweep through a range of BASS round designs, reducing flechette diameter, using tungsten as the major mass component of the BASS round. Constrain your BASS design by assuming it occupies no more than 85% of the entire reference cartridge length, has a tail stock that is at least as long as the nose and goes no faster than Mach 10. Iterate about gun elevation angle, muzzle velocity, BASS round geometry, working to find that design that uses the smallest amount of tungsten at the prescribed range at STP while maintaining the same amount of KE on target as the reference round in Chapter 3.

5. Performance Plots

On the figures prescribed below, plot i.) Reference round from HW3, ii.) Minimum TOF BASS round, iii.) Maximum KE BASS round, iv.) Minimum cost BASS round.

5.1 Plot Altitude vs Range

On a ~half-page figure, plot the altitude of the round in meters on the vertical axis against the range of the round on the horizontal axis in meters for your round from the muzzle to target range.

5.2 Plot Mach vs Range

On a ~half-page figure, plot the Mach of the round on the vertical axis against the range of the round on the horizontal axis in meters for your round from the muzzle to target range.

5.3 Plot Total Speed vs Range

On a ~half-page figure, plot the Total Speed of the round on the vertical axis in m/s against the range of the round on the horizontal axis in meters for your round from the muzzle to target range.

5.4 Plot Kinetic Energy vs Range

On a ~half-page figure, plot the Kinetic Energy in J of the round on the vertical axis against the range of the round on the horizontal axis in meters for your round from the muzzle to target range.

5.5 Plot Elapsed Time vs Range

On a ~half-page figure, plot the Elapsed Time, t (s) of the round on the vertical axis against the range of the round on the horizontal axis in meters for your round from the muzzle to target range.

6. Conclusions and Recommendations

6.1 Conclusions

Draw conclusions about how well your code works.  List the gun elevation angles required for each of the designs and overall conclusions about each round type.

6.2 Recommendations

Make any real (emphasis added) recommendations about your code.

- References

As laid out in Report 1.

Appendices: Include as much information as the team deems relevant in one or more appendices. Make sure to include the appendix number and name in the Table of Contents, but not its page numbers. Note that appendices are stand-alone documents with page numbering like A-1, A-2, A-3… etc.