AE 721 Report 5 Required Contents

- Due Monday 21 October 2024 9am

by e-mail to <kuaerodesign@gmail.com>
AE721_Report5_TeamProjectName.docx	   example: AE721_Report5_20mmBASS.docx

AE721_Report5_TeamProjectName.pdf 	   example: AE721_Report5_20mmBASS.pdf

AE721_Report5_TeamProjectName.xslx (or other code extension)

Team Assignment

Note that the work to be performed for this report is to be done by a team of students.

- Total Points: 50

What follows is a specification for the minimum required contents report. Your report must be organized in this manner. You may, within each section, add sub-sections if you so desire.  Each chapter must start with a statement of purpose, except the Introduction and the Summary/Conclusions chapters.

- Report Title: AE 721 Report 5 (Chosen Interceptor Missile Project) Benchmark L/D Determination

Cover - This is a good idea for both the .doc and .pdf files as it will be necessary when the file is printed to a hard copy.  Name all team members.

Title Page - This page should be the first page of the report (if a cover is used, then it will be within the cover page). This page should be numbered as page "i."

Acknowledgments – Optional, make them real. Thank Mom/Dad/HS Teachers/Pets as you feel appropriate.

Table of Contents - The table of contents shall be structured as follows:

Table of Contents                                 page

List of Symbols	iii

Acknowledgment	iv

1. Introduction	1

2. Chapter 2 Major Heading	3

2.1 Chapter 2 First Section	3

2.2 Chapter 2 Second Section	5

2.2.1 Chapter 2 First Subsection of Second Section	5

etc.

Page Numbering - The first section of the report containing the list of symbols, acknowledgment, list of tables and list of figures etc. shall be numbered with lower case Roman numerals. The body shall be numbered sequentially with Arabic numerals 1, 2, 3, etc. The Appendices shall be numbered with the letter of the appendix first followed by a dash and a page number: A-1, A-2, A-3; B-1, B-2, B-3 etc.

List of Symbols - You must include any symbols that are used in this report. You must give the names of the symbols and the units. After the main body of the symbols, you should include Greek symbols, then subscripts, followed by acronyms.

List of Figures - List any and all figures that are found in the report along with the page they are found on.

List of Tables - List any and all tables that are found in the report along with the page they are found on.

Notes on Generated Codes and Plots

Any suitable code may be used (like Excel, MatLab, other coding language). The original code must be turned in with the homework.

Notes on Previous Work

Include write-ups and technical work from previous assignments. Correct any mistakes made in the new report to be turned in. Note that if an error is flagged in an earlier report and it is not corrected in a later report, twice the number of points will come off.

1. Introduction & Background Research on Benchmark Systems

Research your benchmark missile systems (Table 1 Below). Tell the reader in words what each of the benchmark system does, what its state of research or sales volumes and combat records have been like (if available). Look at Table 2 for more details on what to consider for this section.

Table 1 Team Chosen Missile Systems

Team 1 AIM-9/FIM-92 Interceptor

Team 2 Starstreak Interceptor

Team 3 MASS Interceptor

Team 4 Electric Missile Interceptor

Team 5 RAIDER Missile Interceptor

Team 6 COTS Interceptor

2. Benchmark Descriptions & Salient Characteristics

Research the benchmark systems below and list benchmark salient characteristics of the missile itself including all weights, volumes, warheads, propellents, etc. Also list the performance values for each system.

Table 2 Benchmark Interceptor Team Assignments

| Interceptor Variant | Benchmark |
| --- | --- |
| Team 1 AIM-9/FIM-92 Interceptor | Using publicly available information, determine as much salient information as possible about the AIM-9 and FIM-92 families of missiles. Consider the most modern variants of the missiles. |
| Team 2 Starstreak Interceptor | Using publicly available information, determine as much salient information as possible about the AIM-9 and Starstreak families of missiles. Consider the most modern variants of the missiles. |
| Team 3 MASS Interceptor | Get information on air defense artillery guns that are capable of hitting targets as high as 30,000 ft. Investigate first the German 88mm FlaK guns. List the salient characteristics of the guns and ammo. |
| Team 4 Electric Missile Interceptor | Read 40mm Valkyrie report and list the salient characteristics of the aircraft. Find salient characteristics of the several variants of the Aerovironment Switchblade drones. |
| Team 5 RAIDER Missile Interceptor | Examine last year’s RAIDER AIM-9 and RAIDER AIM-120 reports and list the salient characteristics of each missile. Also list the salient characteristics of more modern AIM-9 and AIM-120 missiles. |
| Team 6 COTS Interceptor | Find as much information as possible describing the COTS 4” dia. Model AMRAAM AIM-120C. Find as much information about the AIM-120C as possible as well. List the salient characteristics of both. |

3. Benchmark Round Performance L/D Determination

For the chosen round family, determine and list its performance numbers. For the following, get all relevant geometries listed in Fleeman to estimate aerodynamic characteristics (lengths, diameters, nose geometries fin spans etc.). Stick as faithfully to the benchmark rounds listed below, adjusting and interpolating for size as needed.

Table 3 Interceptor Team Assignments

| Interceptor Variant | Benchmark |
| --- | --- |
| Team 1 AIM-9/FIM-92 Interceptor | Using the information in Chapter 2 and the class notes, reverse engineer modern AIM-9 family members and FIM-92 family L/Dcruise values. |
| Team 2 Starstreak Interceptor | Using the information in Chapter 2 and the class notes, reverse engineer modern AIM-9 family members and Starstreak family L/Dcruise values. |
| Team 3 MASS Interceptor | Enter each benchmark round found in Chapter 2 in your ballistics code and verify the performance of the system. Provide a plot showing altitude with range at the prescribed muzzle velocity. |
| Team 4 Electric Missile Interceptor | Read 40mm Valkyrie report and determine the design L/Dcruise value for the RAIDER and if possible, the Switchblade family of munitions. |
| Team 5 RAIDER Missile Interceptor | Using the information in Chapter 2 and the class notes, reverse engineer modern AIM-9 family members and AIM-120 family L/Dcruise values. |
| Team 6 COTS Interceptor | Using the information in Chapter 2 and the class notes, reverse engineer modern AIM-120 family L/Dcruise values. |

4. Conclusions and Recommendations

4.1 Conclusions

Draw conclusions about your baseline weapon system’s overall performance, lift-to-drag ratio. Include information from all sections in this document including the ones from previous homework assignments.

4.2 Recommendations

Make any real (emphasis added) recommendations about the benchmark weapon system, analysis or modifications that could be made.

- 5. References

As laid out in Report 1.

Appendices: Include as much information as the team deems relevant in one or more appendices. Make sure to include the appendix number and name in the Table of Contents, but not its page numbers. Note that appendices are stand-alone documents with page numbering like A-1, A-2, A-3… etc. Include a Table of Contents in the appendix if it is over 5 pages long.  Include also symbols that are used in the appendices in a List of Symbols in the appendix if the number of pages is over 5 pages long.

Nota Bene: If Matlab or another computational code is used to generate figures in the body, include the code in a separate appendix, one appendix for each code used. If Excel is used, then include screen shots. Note that both Matlab and Excel (and any other computational codes) must be submitted separately as well.